Here’s a technician-oriented reference for common Bosch appliance error codes and what they usually mean in the field. Bosch appliances are generally considered premium platforms, but they rely heavily on:

- Heat pumps

- Flow meters

- Leak protection systems (AquaStop)

- Thermistors

- Electronic communication systems

Bosch also tends to use more European-style diagnostics than Whirlpool/GE platforms. Their dishwashers especially have extensive fault reporting. Official Bosch error references are centralized here: Bosch Error Code Support

Bosch Dishwasher Error Codes

Bosch dishwashers are by far the most commonly serviced Bosch appliance category in the U.S.

| Code | Meaning | Most Common Causes | Recommended Fix |
| --- | --- | --- | --- |
| E01 | Heat pump circuit fault | Main control or heat pump issue | Reset power, inspect heat pump |
| E02 / E03 | Main relay/control fault | PCB relay failure | Inspect/replace control board |
| E04 | Heater electronics fault | Heater circuit failure | Test heater electronics |
| E05 | Diverter circuit fault | Wash arm diverter issue | Inspect diverter motor |
| E06 | Door mechanism/sensor issue | Door latch or sensor fault | Check latch and alignment |
| E07 | Drying fan fault | Zeolite fan blocked or failed | Inspect drying fan |
| E09 | Heater failure | Heat pump heater open | Test heater continuity |
| E11 | Thermistor/temp sensor issue | Failed NTC sensor | Replace thermistor |
| E12 | Limescale buildup in heat pump | Hard water scaling | Descale dishwasher |
| E14 | Flow meter/water fill issue | Restricted fill or bad flow meter | Inspect inlet and flow meter |
| E15 | AquaStop leak detected | Water in base pan | Locate leak and dry base |
| E16 / E17 | Water fill problem | Inlet restriction or valve issue | Check inlet hose and valve |
| E18 | Low water level | Weak supply pressure | Verify water flow |
| E19 | Detergent dispenser fault | Solenoid issue | Inspect dispenser assembly |
| E20 / E21 | Heat pump blocked/fault | Circulation pump issue | Inspect heat pump |
| E22 | Filter blocked | Dirty sump filter | Clean filter thoroughly |
| E23 | Drain pump issue | Blocked or failed drain pump | Inspect pump |
| E24 | Drain restriction | Kinked hose or blocked drain | Check hose and air gap |
| E25 | Drain pump blocked | Debris in pump | Remove obstruction |
| E27 | Low incoming voltage | House voltage issue | Verify supply voltage |
| E30 | AquaStop flood mode | Leak or water in base tray | Dry base and inspect leaks |

Bosch officially identifies E15 as water detected in the base tray activating the leak protection system.

The E24 and E25 drain errors are among the most common Bosch dishwasher service calls.

Bosch Washer Error Codes

| Code | Meaning | Most Common Causes | Recommended Fix |
| --- | --- | --- | --- |
| E16 / F16 | Door not locked | Failed latch or door not shut | Check door lock |
| E17 / F17 | Fill timeout | Low water pressure | Inspect inlet system |
| E18 / F18 | Drain timeout | Restricted drain or weak pump | Clean pump/filter |
| E23 / F23 | AquaStop activated | Leak detected in base | Inspect for leaks |
| E29 | Water inlet problem | Restricted inlet flow | Check supply and screens |
| E32 | Unbalanced load | Suspension or load issue | Redistribute load |
| E36 | Door lock problem | Failed interlock | Replace lock |
| E57 | Inverter module issue | Motor control failure | Test inverter board |
| E67 | Control module fault | PCB communication issue | Inspect control boards |

Bosch laundry platforms commonly use inverter-driven motors and sophisticated load balancing systems.

Bosch Dryer Error Codes

| Code | Meaning | Most Common Causes | Recommended Fix |
| --- | --- | --- | --- |
| E01 | Door issue | Door switch or latch | Inspect latch |
| E03 | Drain issue (condensing models) | Condensate blockage | Clean drain system |
| E06 | Heating problem | Heater or thermistor issue | Test heating circuit |
| E09 | Heat pump issue | Heat pump failure | Inspect sealed system |
| E11 | Thermistor fault | Failed temp sensor | Replace thermistor |
| E90 | Communication error | PCB/UI communication fault | Check harness |

Bosch heat-pump dryers require extremely regular maintenance, especially lint and condensate cleaning. Techs often see pump and moisture-related failures from neglected maintenance.

Bosch Refrigerator Error Codes

| Code | Meaning | Most Common Causes | Recommended Fix |
| --- | --- | --- | --- |
| E01 | Refrigerator sensor fault | Failed thermistor | Test sensor |
| E02 | Freezer sensor fault | Thermistor failure | Replace sensor |
| E03 | Defrost sensor issue | Defrost thermistor fault | Test sensor |
| E10 | Evaporator fan problem | Failed evap fan motor | Inspect fan |
| E15 | Ice maker fault | Ice maker communication issue | Inspect ice maker |
| E20 | Defrost system issue | Heater or thermistor fault | Test defrost system |
| E21 | Compressor communication issue | Inverter or compressor fault | Inspect inverter board |
| E30 | Main communication fault | UI/main PCB mismatch | Check wiring/connectors |

Bosch refrigeration platforms heavily rely on communication between inverter compressors and main control boards.

Bosch Oven / Range Error Codes

| Code | Meaning | Most Common Causes | Recommended Fix |
| --- | --- | --- | --- |
| E011 | Stuck keypad | Failed touch control | Replace UI |
| E115 | Overtemperature | Cooling fan or relay issue | Inspect fan |
| E305 | Door lock issue | Self-clean latch problem | Test latch motor |
| E513 | Communication failure | PCB communication issue | Inspect harness |
| E9000 | Voltage supply issue | Incoming power issue | Verify voltage |
| E9050 | Main PCB fault | Control board failure | Replace control |

Bosch Microwave Error Codes

| Code | Meaning | Most Common Causes | Recommended Fix |
| --- | --- | --- | --- |
| E100 | Communication issue | UI/main board problem | Inspect connectors |
| E101 | Door switch issue | Interlock fault | Test switches |
| E104 | Magnetron overheating | Cooling airflow issue | Inspect cooling fan |
| E105 | Temp sensor issue | Thermistor failure | Replace sensor |

Common Bosch Failure Patterns

Dishwashers

- Heat pumps

- Drain restrictions

- AquaStop leak faults

- Flow meters

- Zeolite drying fans

Laundry

- Inverter boards

- Door locks

- Drain pumps

- Water leak sensors

Refrigeration

- Thermistors

- Evaporator fans

- Inverter compressor communication

Cooking

- Cooling fans

- Touch controls

- Door lock assemblies

Bosch Diagnostic Tips

- Bosch appliances are very sensitive to voltage irregularities.

- AquaStop leak detection is extremely common:

  - Even tiny leaks trigger E15/F23-style faults.

- Bosch dishwashers frequently clog at:

  - Filter

  - Pump cover

  - Air gap

  - Garbage disposal knockout

- Heat-pump systems dominate Bosch premium appliances:

  - Dishwashers

  - Dryers

- Many Bosch errors clear after disconnecting power for 2–5 minutes.