Here’s a technician-oriented reference for common Frigidaire appliance error codes, what they mean, and the most common repair paths. This also applies to many products from Frigidaire and related Electrolux platforms, since Frigidaire is owned by Electrolux. You’ll also see similar code structures on some Kenmore models manufactured by Frigidaire/Electrolux.

Frigidaire Washer Error Codes

| Code | Meaning | Most Common Causes | Recommended Fix |
| --- | --- | --- | --- |
| E11 | Fill timeout | Low water pressure or clogged screens | Check inlet valves and screens |
| E13 | Water leak detected | Water in base pan | Inspect for leaks and dry base |
| E21 | Long drain | Clogged drain hose or weak pump | Clean filter and test pump |
| E23 | Drain pump relay failure | Main board issue | Inspect control board |
| E24 | Drain relay sensing issue | PCB fault | Check control board |
| E31 | Pressure sensor fault | Bad pressure switch or hose | Inspect hose and sensor |
| E35 | Overflow condition | Stuck inlet valve or pressure issue | Unplug washer and inspect valves |
| E41 | Door open | Door not closed properly | Check latch and strike |
| E43 | Door lock circuit issue | Failed door lock or triac | Test lock assembly |
| E47 | Door lock sensing failure | Control not seeing lock | Inspect harness and PCB |
| E52 | No tachometer signal | Motor or hall sensor fault | Check motor windings |
| E57 | High motor current | Locked motor or inverter issue | Test motor and inverter |
| E58 | High motor current to inverter | Inverter failure | Inspect MCU/inverter |
| E5A | Heat sink overheating | Motor control overheating | Check cooling and drive load |
| E66 | Heater relay fault | Heater circuit or PCB issue | Test heater relay |
| E71 | Thermistor open | Temp sensor failed | Replace thermistor |
| E74 | Wash temp out of range | Sensor placement or failure | Inspect thermistor |
| EF1 | Drain filter blocked | Restricted drain system | Clean filter |
| EF2 | Oversudsing | Excess detergent | Flush washer and reduce detergent |

Frigidaire/Electrolux front-load washers heavily rely on inverter motor systems and pressure sensing, so E5x-series motor faults are very common. (frigidaire.com)

Frigidaire Dryer Error Codes

| Code | Meaning | Most Common Causes | Recommended Fix |
| --- | --- | --- | --- |
| E24 | Thermistor open | Failed temp sensor | Test thermistor |
| E25 | Thermistor shorted | Sensor short | Replace thermistor |
| E4A | Heating issue | Heater relay or element problem | Inspect heater circuit |
| E5B | Line voltage problem | Low/high supply voltage | Verify power supply |
| E61 | Heater timeout | Weak heater output | Test heater element |
| E64 | Heater circuit open | Failed heater | Check continuity |
| E68 | Stuck button | UI/control issue | Inspect keypad |
| EAF | Restricted airflow | Blocked venting | Clean vent system |
| LOC | Control lock enabled | Child lock active | Disable lock |
| HO | Overheating | Airflow or cycling issue | Inspect venting and thermostats |

Restricted airflow is one of the biggest causes of Frigidaire dryer heating faults and EAF errors. (repairclinic.com)

Frigidaire Refrigerator Error Codes

| Code | Meaning | Most Common Causes | Recommended Fix |
| --- | --- | --- | --- |
| SY CE | Communication failure | UI/main board communication | Check harness/connectors |
| SY EF | Evaporator fan circuit fault | Failed evap fan | Test fan motor |
| OP | Freezer door open | Door ajar | Close door/check switch |
| SH | Shorted thermistor | Temp sensor short | Replace thermistor |
| 5Y CF | Main control communication fault | PCB issue | Inspect control board |
| dF | Defrost failure | Heater or thermistor fault | Test defrost system |
| HI TEMP | High temp alarm | Cooling problem | Check airflow and sealed system |
| ER IF | Ice maker fan fault | Frozen or failed fan | Defrost and test fan |
| ER CE | Communication error | PCB communication failure | Inspect wiring |
| ER SY | System communication fault | Main board issue | Test controls |

Frigidaire refrigerators are well known for evaporator fan, thermistor, and communication-board failures on electronic models. (reddit.com)

Frigidaire Dishwasher Error Codes

| Code | Meaning | Most Common Causes | Recommended Fix |
| --- | --- | --- | --- |
| i10 | Fill problem | Low water supply | Check inlet valve |
| i20 | Drain issue | Restricted drain or weak pump | Clean hose/filter |
| i30 | Leak/flood detected | Water in base pan | Dry pan and locate leak |
| i40 | Pressure sensor issue | Faulty pressure sensor | Test pressure switch |
| i50 | Wash motor problem | Failed circulation motor | Test wash pump |
| i60 | Heating issue | Heater or thermistor fault | Inspect heater circuit |
| iC0 | Communication failure | UI/main board issue | Check wiring |
| UF | Vent fan issue | Vent assembly failure | Inspect vent fan |
| CL | Drain pump running continuously | Flood switch active | Check base tray |

Frigidaire dishwashers commonly throw i30 flood codes from even small amounts of water reaching the base float tray. (frigidaire.com)

Frigidaire Oven / Range Error Codes

| Code | Meaning | Most Common Causes | Recommended Fix |
| --- | --- | --- | --- |
| F1 | Electronic control failure | Bad ERC/control board | Replace control |
| F10 | Runaway temperature | Stuck relay or RTD issue | Test sensor and relay |
| F11 | Stuck keypad | Touchpad short | Replace keypad |
| F13 | Control memory issue | EEPROM corruption | Replace board |
| F30 | Oven sensor open | Failed RTD sensor | Test resistance |
| F31 | Oven sensor shorted | Shorted RTD | Replace sensor |
| F90 | Door lock motor failure | Failed latch assembly | Inspect lock motor |
| F91 | Door latch switch issue | Latch switch not sensing | Test switches |
| F95 | Cooling fan issue | Failed cooling fan | Test fan motor |

Frigidaire ranges are especially known for F10 runaway temperature faults caused by failing relays or RTD sensors. (appliancepartspros.com)

Frigidaire Microwave Error Codes

| Code | Meaning | Most Common Causes | Recommended Fix |
| --- | --- | --- | --- |
| F1 | Control board issue | Failed PCB | Replace control |
| F3 | Humidity sensor problem | Sensor fault | Replace sensor |
| F5 | Door switch issue | Interlock failure | Test switches |
| SE | Stuck keypad | Membrane short | Replace touchpad |
| E-11 | Temp sensor issue | Thermistor failure | Test sensor |

Common Frigidaire Failure Patterns

Laundry

- Door lock assemblies

- Inverter boards

- Pressure sensors

- Drain pumps

Refrigeration

- Evaporator fans

- Main boards

- Thermistors

- Ice maker fan systems

Dishwashers

- Flood switches

- Circulation pumps

- Pressure sensors

Cooking

- RTD temperature sensors

- Electronic control boards

- Door latch motors

Frigidaire Diagnostic Tips

- Many Frigidaire/Electrolux appliances store fault history in service mode.

- Frigidaire commonly uses:

  - Thermistors

  - Pressure sensors

  - Inverter motor controls

  - Flood/base leak switches

- Communication errors are very common on refrigerator platforms.

- Many “mystery” dishwasher drain problems are actually triggered flood switches underneath the tub.