ASSOCIATE ALL GE FAMILY BRANDS WITH GE WHEN SEARCHING

Here’s a technician-focused reference for common GE appliance error codes across washers, dryers, refrigerators, dishwashers, ovens/ranges, and microwaves. This also applies to many products from GE Appliances, including some Hotpoint and Café platforms. GE error structures vary a lot by generation, especially between older hydrowave/VMW-style laundry units and newer inverter/UI-based appliances.

GE Washer Error Codes

| Code | Meaning | Most Common Causes | Recommended Fix |
| --- | --- | --- | --- |
| E22 | Fill timeout | Low water pressure, clogged screens | Check supply valves and inlet screens |
| E23 | Drain timeout | Clogged pump/filter | Inspect drain pump and hose |
| E30 | Door lock failure | Bad latch or harness | Test door lock assembly |
| E31 | Door unlock failure | Failed lock mechanism | Inspect latch and control |
| E38 | Lid lock failure | Top-load lid lock issue | Test lid lock and strike |
| E39 | Lid switch fault | Open lid switch circuit | Check continuity of switch |
| E43 | Motor control fault | Motor inverter/MCU issue | Inspect motor and inverter board |
| E45 | Control board communication | UI/main board mismatch | Reseat connectors |
| E46 | Water level sensor issue | Pressure sensor/hose problem | Inspect pressure hose |
| E54 | Motor overload | Basket drag or failing motor | Check tub movement and motor |
| E57 | High motor current | Locked rotor or shorted motor | Test motor windings |
| E58 | Motor overtemp | Overloaded or failing inverter | Check cooling and motor load |
| E61 | Heater timeout | Failed heater or thermistor | Test heating circuit |
| E66 | Water heater relay fault | Main board issue | Inspect heater relay |
| E71 | Thermistor open | Failed temperature sensor | Test sensor resistance |
| E72 | Thermistor out of range | Sensor drifting | Replace thermistor |
| E90 | UI communication fault | UI/main PCB communication | Check harness and boards |

Many newer GE front-load washers use inverter-driven motors, and E43–E58 motor-related codes often point to inverter boards or hall/motor feedback issues. (geappliances.com)

GE Top-Load Washer Specific Codes

| Code | Meaning | Common Causes | Recommended Fix |
| --- | --- | --- | --- |
| Lid | Lid open | Lid not closed | Close lid |
| H2O SUPPLY | Water supply issue | Low water or closed valve | Check supply |
| Balance | Out-of-balance load | Suspension wear | Redistribute load |
| Fill | Fill problem | Inlet restriction | Inspect valves |
| Drain | Drain problem | Pump restriction | Check pump/hose |

GE Dryer Error Codes

| Code | Meaning | Most Common Causes | Recommended Fix |
| --- | --- | --- | --- |
| E1 | Thermistor open | Failed thermistor | Test/replace thermistor |
| E2 | Thermistor shorted | Sensor short | Replace thermistor |
| E3 | Airflow restriction | Blocked venting | Clean vent system |
| E4 | Humidity sensor issue | Sensor contamination | Clean/test sensor bars |
| E5 | Control board issue | Main PCB fault | Replace board |
| E6 | Motor fault | Motor relay or winding issue | Test motor |
| E7 | Door switch issue | Failed switch | Inspect door switch |
| PF | Power failure | Voltage interruption | Restart cycle |
| F01 | Main control failure | PCB corruption | Replace control |
| F70/F71 | UI communication issue | Ribbon cable or UI fault | Reseat connectors |

GE dryers are extremely sensitive to vent restrictions, and E3 airflow-related faults commonly result from partially restricted venting rather than failed heating components. (repairclinic.com)

GE Refrigerator Error Codes

| Code | Meaning | Most Common Causes | Recommended Fix |
| --- | --- | --- | --- |
| FF | Fresh food thermistor issue | Failed sensor | Test thermistor |
| PF | Power failure | Recent outage | Reset alarm |
| CI | Ice maker communication fault | Ice maker PCB issue | Check harness |
| dE | Defrost error | Heater or thermistor failure | Test defrost system |
| CC | Communication error | UI/main board mismatch | Inspect connectors |
| HRS | High refrigerator temp | Cooling issue | Check airflow and sealed system |
| tC | Turbo cool active | Normal operation | No repair needed |
| SY EF | Evaporator fan error | Failed evap fan motor | Test fan |
| 88 88 | Power-up diagnostic | Board reboot state | Usually normal after outage |
| Er | General board fault | PCB issue | Inspect controls |

GE refrigerators frequently experience evaporator fan, thermistor, and communication-related failures on electronic models. (reddit.com)

GE Dishwasher Error Codes

| Code | Meaning | Most Common Causes | Recommended Fix |
| --- | --- | --- | --- |
| C1 | Drain timeout | Clogged drain | Clean hose/filter |
| C2 | Drain pump issue | Pump failure | Test drain pump |
| C3 | Water temperature issue | Heater not operating | Check heater |
| C4 | Float switch activated | Leak or overfill | Inspect float/base |
| C5 | Fill timeout | Water supply issue | Check inlet valve |
| C6 | Water temp sensor issue | Thermistor fault | Test sensor |
| C7 | Heating circuit issue | Failed heater relay | Inspect PCB |
| 815 | UI communication issue | Ribbon cable fault | Reseat UI |
| LEAK DETECTED | Base water sensor active | Water in pan | Dry pan and locate leak |

GE dishwashers commonly enter drain-only safety mode when the float switch or leak sensor is triggered. (geappliances.com)

GE Oven / Range Error Codes

| Code | Meaning | Most Common Causes | Recommended Fix |
| --- | --- | --- | --- |
| F0 | Keypad stuck | Failed touchpad | Replace keypad |
| F1 | Control board failure | ERC/main PCB issue | Replace control |
| F2 | Oven overtemperature | Stuck relay or sensor | Test RTD and relays |
| F3 | Oven sensor open | Failed RTD sensor | Test resistance |
| F4 | Oven sensor shorted | Shorted RTD | Replace sensor |
| F5 | Door latch issue | Failed latch motor | Inspect latch |
| F7 | Function key stuck | Touchpad failure | Replace UI |
| FC | Door lock circuit problem | Lock motor/switch fault | Test latch assembly |
| FD | Meat probe issue | Probe short/open | Test probe |

GE ranges are known for F7 stuck-key faults and RTD sensor-related F3/F4 errors. (appliancepartspros.com)

GE Microwave Error Codes

| Code | Meaning | Most Common Causes | Recommended Fix |
| --- | --- | --- | --- |
| F1 | Control board issue | Failed PCB | Replace control |
| F2 | Keypad short | Membrane switch fault | Replace touchpad |
| F3 | Humidity sensor issue | Sensor fault | Replace humidity sensor |
| F4 | Open temperature sensor | Thermistor failure | Test sensor |
| F5 | Door switch issue | Interlock switch failure | Test switches |
| SE | Stuck keypad | Shorted touchpad | Replace UI |

Common GE Appliance Failure Patterns

Laundry

- Inverter boards

- Lid locks

- Pressure sensors

- Drain pumps

Refrigeration

- Evaporator fans

- Thermistors

- Main board communication failures

- Ice maker electronics

Dishwashers

- Flood switches

- Wash pumps

- Heater circuits

Cooking

- Touchpads/keypads

- RTD oven sensors

- Door latch motors

GE Diagnostic Tips

- GE often stores hidden fault history in service mode.

- Many GE appliances use thermistors instead of traditional thermostats.

- Communication faults are extremely common on newer GE electronic platforms.

- GE inverter laundry systems often require checking:

  - Hall feedback

  - Stator winding resistance

  - Inverter DC voltage