LG appliances heavily rely on:

Hall sensors

Inverter boards

Thermistors

Leak/flood switches

Communication between UI and main PCB

Because of that, a lot of LG diagnostics center around inverter communication and sensor feedback.

LG Washer Error Codes

Code	Meaning	Most Common Causes	Recommended Fix

IE	Water inlet error	Low water pressure, clogged screens	Check inlet valves and filters

OE	Drain error	Clogged drain filter or weak pump	Clean filter and inspect pump

UE / uE	Unbalanced load	Off-balance basket or worn suspension	Redistribute load, inspect suspension

dE	Door error	Door not closed or failed latch	Check latch and strike

LE	Motor lock error	Hall sensor, stator, or rotor issue	Test hall sensor and stator

FE	Overflow error	Stuck inlet valve or pressure sensor	Inspect inlet valve and pressure system

PE	Pressure sensor error	Faulty water level sensor	Check pressure hose and sensor

tE	Thermistor error	Failed temperature sensor	Test thermistor resistance

CE	Current error	Locked motor or inverter issue	Inspect inverter and motor

CL	Child lock enabled	Feature activated	Disable child lock

PF	Power failure	Voltage interruption	Restart cycle

AE	Leak detected	Water in base pan	Inspect for leaks

vS	Vibration sensor error	Failed vibration sensor	Replace sensor

Sud / Sd	Oversudsing	Too much detergent	Run rinse cycles

E6	Clutch/motor issue	Clutch assembly failure	Inspect clutch and motor

FF	Frozen drain system	Drain hose frozen	Thaw drain line

TE1/TE2	Thermistor variation	Sensor out of range	Replace thermistor

LG washer IE/OE/UE faults are among the most common service calls, while LE motor lock faults frequently trace back to failed hall sensors or stator issues.

LG Dryer Error Codes

Code	Meaning	Most Common Causes	Recommended Fix

d80 / d90 / d95	Restricted venting	Blocked exhaust duct	Clean vent system completely

tE1 / tE2 / tE4	Thermistor fault	Failed temp sensor	Test thermistor

dE	Door open error	Door switch issue	Inspect door switch

LE1	Motor locked	Drive motor issue	Test motor windings

AE / LE2	Compressor issue (heat pump models)	Compressor fault	Inspect compressor/inverter

OE	Drain error (combo units)	Drain restriction	Inspect pump and hose

PF	Power failure	Voltage interruption	Restart dryer

HS	Humidity sensor issue	Dirty sensor bars	Clean moisture sensors

nP	Incorrect voltage	Missing L2 voltage	Verify 240V supply

PS	Power supply issue	Voltage irregularity	Check outlet and breaker

CL	Child lock	Lock feature active	Disable lock

F1	Drum overheating	Restricted airflow	Clean venting

LG dryers are notorious for very sensitive FlowSense vent detection systems. Even moderately long duct runs can trigger d80/d90/d95 errors.

LG Refrigerator Error Codes

Code	Meaning	Most Common Causes	Recommended Fix

ER IF	Ice fan error	Frozen or failed ice fan	Defrost and test fan

ER FF	Evaporator fan failure	Failed evap fan motor	Test fan motor

ER CF	Communication fan error	PCB/fan communication issue	Inspect harness

ER CO	Communication error	UI/main board failure	Check connectors

ER DH	Defrost heater failure	Heater or thermistor issue	Test defrost circuit

ER dS	Defrost sensor issue	Bad thermistor	Replace sensor

ER FS	Freezer sensor error	Faulty thermistor	Test sensor

ER IS	Ice maker sensor fault	Ice maker fan or sensor issue	Inspect ice maker

ER SS	Pantry sensor issue	Sensor failure	Replace thermistor

ER RT	Room temp sensor issue	Ambient thermistor fault	Test sensor

ER HS	Humidity sensor issue	Sensor or PCB issue	Inspect control

FF	Unit detecting warm temps	Cooling problem	Check sealed system

OFF / O FF	Demo mode active	Showroom mode enabled	Disable demo mode

Older LG refrigerators became infamous for linear compressor failures, though newer models reportedly improved somewhat.

LG Dishwasher Error Codes

Code	Meaning	Most Common Causes	Recommended Fix

IE	Fill error	Low water supply	Check inlet valve

OE	Drain error	Blocked drain hose or pump	Clean drain path

AE / E1	Leak detected	Water in base tray	Dry pan and locate leak

bE	Button or detergent issue	Stuck key or oversudsing	Check detergent and UI

CE	Motor current error	Wash motor overload	Test circulation pump

dE	Door error	Door latch problem	Inspect latch

FE	Overflow error	Overfill condition	Inspect inlet valve

HE	Heater error	Heater circuit failure	Test heater

LE	Motor error	Wash motor issue	Inspect circulation pump

tE	Thermal error	Thermistor problem	Replace thermistor

CL	Child lock	Lock enabled	Disable lock

PF	Power interruption	Voltage loss	Restart cycle

LG dishwashers commonly throw AE/E1 leak codes from even tiny amounts of moisture reaching the base leak sensor.

LG Oven / Range Error Codes

Code	Meaning	Most Common Causes	Recommended Fix

F1	Oven sensor open	Failed RTD sensor	Test resistance

F2	Oven sensor shorted	Shorted RTD	Replace sensor

F3	Keypad short	Failed touchpad	Replace UI

F6	Oven overheating	Cooling fan or relay issue	Check relays and fan

F9	Failed preheat	Weak bake circuit	Test igniter/heater

F10	Door lock motor failure	Failed latch motor	Inspect latch

F11	PCB communication fault	Main/UI communication	Check harness

CF1–CF8	Cooktop sensor/communication faults	Induction PCB issues	Inspect sensors and boards

CFA	Overcurrent fault	Power issue	Check incoming voltage

CFC	Overvoltage error	Voltage spike	Verify supply voltage

LG induction ranges use a large number of sensor-based CF-series diagnostics for IGBT and burner communication faults.

LG Microwave Error Codes

Code	Meaning	Most Common Causes	Recommended Fix

F3	Humidity sensor issue	Failed sensor	Replace humidity sensor

F5	Door switch issue	Failed interlock switch	Test switches

SE	Keypad short	Stuck membrane keypad	Replace touchpad

E-11	Temperature sensor issue	Thermistor failure	Test sensor

LOC	Control lock active	Lock feature enabled	Disable lock

Common LG Failure Patterns

Laundry

Hall sensors

Rotor/stator assemblies

Drain pumps

Vibration sensors

Inverter boards

Refrigeration

Linear compressors (older models)

Evaporator fans

Ice room fan systems

Communication boards

Dishwashers

Leak sensors

Wash motors

UI/control glitches

Cooking

RTD sensors

Keypads

Induction communication boards

LG Diagnostic Tips

LG service mode often stores hidden historical faults.

Many LG appliances can be reset by unplugging for 5 minutes.

LG uses thermistors almost everywhere instead of traditional thermostats.

Hall sensor failures are extremely common in LG direct-drive washers.

Venting issues account for a huge percentage of LG dryer service calls.