Master Cross-Brand Appliance Diagnostic Chart (Field Reference)

This is a consolidated field-tech quick reference across the major appliance brands:

- Whirlpool

- Samsung

- LG Electronics

- GE Appliances

- Frigidaire

- Bosch

This is organized by symptom/failure type rather than manufacturer-specific code systems, which is usually how real field diagnosis works.

1. Washer Diagnostic Cross-Reference

| Symptom | Whirlpool/Maytag | Samsung | LG | GE | Frigidaire | Bosch | Most Common Actual Cause |
| --- | --- | --- | --- | --- | --- | --- | --- |
| No Fill | F8E1 / LF | 4C / 4E | IE | E22 | E11 | E17 | Closed valve, clogged screens |
| No Drain | F9E1 | 5C / 5E | OE | E23 | E21 | E18 | Drain pump/filter blockage |
| Door Lock | F5E2 | dC / dE | dE | E30 | E41 | F16 | Failed latch assembly |
| Unbalanced | UL / UB | UE / UB | UE | Balance | E4x | E32 | Suspension wear |
| Oversuds | Sd | Sud | Sud | — | EF2 | — | Wrong detergent |
| Pressure Sensor | F3E1 | 1C | PE | E46 | E31 | — | Pressure hose blockage |
| Motor/Hall Sensor | F7E1 | 3E | LE | E57 | E52 | E57 | Hall sensor or inverter |
| Overflow | F8E3 | OC / OE | FE | — | E35 | — | Stuck inlet valve |
| Thermistor | F3E1 | tE | tE | E71 | E71 | E11 | Failed temp sensor |
| Leak Detection | — | LC | AE | — | E13 | F23 | Water in base pan |

2. Dryer Diagnostic Cross-Reference

| Symptom | Whirlpool | Samsung | LG | GE | Frigidaire | Bosch | Most Common Cause |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Restricted Vent | AF / F4E3 | CL9 / HC | d80/d90/d95 | E3 | EAF | — | Blocked venting |
| Thermistor Fault | F22/F23 | tE | tE1/tE2 | E1/E2 | E24/E25 | E11 | Failed temp sensor |
| Door Switch | F29 | dE | dE | E7 | — | E01 | Failed switch |
| Heating Failure | F20 | HC | HS | — | E61/E64 | E06 | Heater open |
| Voltage Issue | L2 | 9C1 | nP/PS | PF | E5B | E27 | Supply voltage problem |
| Motor Error | — | 3E | LE1 | E6 | — | — | Motor/inverter issue |

3. Refrigerator Diagnostic Cross-Reference

| Symptom | Whirlpool | Samsung | LG | GE | Frigidaire | Bosch | Most Common Cause |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Evap Fan Failure | E2/F2 | 22E | ER FF | SY EF | ER IF | E10 | Failed evaporator fan |
| Defrost Failure | DF | 5E | ER DH | dE | dF | E20 | Heater or thermistor |
| Communication Error | SY CE | PC ER | ER CO | CC | SY CE | E30 | PCB communication |
| Ice Maker Fault | E5/F5 | 41E | ER IS | CI | ER IF | E15 | Ice maker/fan issue |
| Demo Mode | — | OF OF | O FF | tC sometimes mistaken | — | — | Demo mode enabled |
| Thermistor Fault | E1/F1 | 5E | ER FS | FF | SH | E01/E02 | Failed sensor |
| Compressor/Inverter | — | 84C | — | — | — | E21 | Inverter/compressor issue |

4. Dishwasher Diagnostic Cross-Reference

| Symptom | Whirlpool | Samsung | LG | GE | Frigidaire | Bosch | Most Common Cause |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Leak/Flood | F8E4 | LC | AE/E1 | C4 | i30 | E15 | Water in base tray |
| Drain Failure | F9E1 | 5C | OE | C1/C2 | i20 | E24/E25 | Drain blockage |
| Fill Failure | F6E4 | 4C | IE | C5 | i10 | E17 | Inlet issue |
| Heater Fault | F7E1 | HE | HE | C7 | i60 | E09 | Heater/relay issue |
| Wash Motor Fault | F4E3 | 3E | LE/CE | — | i50 | E20/E21 | Circulation pump |
| Door Error | F5E1 | dE | dE | — | — | E06 | Door latch |
| Communication Error | F2E2 | AC | — | 815 | iC0 | E01/E02 | UI/main board |

5. Oven / Range Diagnostic Cross-Reference

| Symptom | Whirlpool | Samsung | LG | GE | Frigidaire | Bosch | Most Common Cause |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Stuck Keypad | F2E0 | SE | F3 | F0/F7 | F11 | E011 | Failed touchpad |
| Oven Sensor Open | F3E0 | tE | F1 | F3 | F30 | — | RTD sensor open |
| Oven Sensor Shorted | F3E1 | — | F2 | F4 | F31 | — | RTD short |
| Door Lock Failure | F5E1 | C-d0 | F10 | FC | F90 | E305 | Latch motor/switch |
| Overtemperature | — | C21 | F6 | F2 | F10 | E115 | Relay or fan issue |
| Communication Error | F6E0 | LE | F11 | — | — | E513 | PCB communication |

6. Microwave Diagnostic Cross-Reference

| Symptom | Whirlpool | Samsung | LG | GE | Frigidaire | Bosch | Most Common Cause |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Door Switch Fault | Door | C-d0 | F5 | F5 | F5 | E101 | Interlock switch |
| Stuck Keypad | F2 | SE | SE | SE | SE | — | Membrane short |
| Temp Sensor Fault | — | E24 | E11 | F4 | E11 | E105 | Thermistor |
| Communication Error | F6 | C-10 | — | — | — | E100 | PCB communication |

Cross-Brand “Usually This Part” Cheat Sheet

| Symptom | Usually the Problem |
| --- | --- |
| Washer won’t spin but drains | Lid lock, hall sensor, actuator |
| Washer hums but won’t drain | Drain pump jammed |
| Dryer heats poorly | Restricted vent |
| Dryer runs no heat | Heater or thermal fuse |
| Fridge warm freezer cold | Evap fan or defrost |
| Fridge dead after storm | Main PCB |
| Dishwasher runs drain constantly | Flood switch/base water |
| Dishwasher fills then stops | Wash pump |
| Oven runaway temp | RTD or stuck relay |
| Microwave dead after door slam | Interlock switches |

Universal Diagnostic Order (Real Tech Workflow)

Most experienced techs follow roughly this order:

Step 1 — Verify Incoming Power

- Proper voltage?

- Loose neutral?

- GFCI tripped?

- Burned terminal block?

Step 2 — Check Stored Error Codes

- Service mode

- Fault history

- Active faults

Step 3 — Mechanical Basics

- Drain restrictions

- Venting

- Water flow

- Door/lid closure

- Fan operation

Step 4 — Sensors

- Thermistors

- Pressure sensors

- Hall sensors

- Float switches

Step 5 — Loads

- Pumps

- Motors

- Heaters

- Compressors

Step 6 — Boards Last

A huge percentage of unnecessary parts replacement comes from replacing boards before verifying:

- power

- harnesses

- sensors

- loads

Universal Resistance Reference Cheat Sheet

Common Thermistor Values

Many appliances use:

- ~10kΩ @ room temp

- ~50kΩ cold

- ~2–3kΩ hot

LG/Samsung/GE/Frigidaire especially use these heavily.

Heater Elements

| Appliance | Typical Resistance |
| --- | --- |
| Dryer heater | 8–30Ω |
| Dishwasher heater | 10–30Ω |
| Oven bake element | 15–40Ω |
| Defrost heater | 20–150Ω |

Most Common Brand-Specific Failure Patterns

| Brand | Common Failures |
| --- | --- |
| Samsung | DC/communication/UI issues |
| LG | Hall sensors, compressors |
| Whirlpool | Lid locks, actuators |
| GE | Boards, inverter laundry |
| Frigidaire | Door locks, fan faults |
| Bosch | AquaStop leaks, heat pumps |