Samsung Washer Error Codes

| Code | Meaning | Most Common Causes | Recommended Fix |
| --- | --- | --- | --- |
| 1E / 1C | Water level sensor error | Pressure sensor issue, clogged hose | Check pressure hose for blockage, inspect sensor |
| LE / LC / LC1 | Leak detected | Water leak, oversudsing | Inspect for leaks, reduce detergent, dry base pan |
| 4E / 4C | Water supply error | Closed valve, clogged inlet screens | Open valves, clean inlet screens, check hoses |
| 5E / 5C / nd | Drain error | Clogged pump filter or drain hose | Clean drain filter, inspect drain pump |
| OE / OC / 0C | Overflow error | Stuck inlet valve or pressure sensor | Unplug unit, inspect inlet valves and sensor |
| UE / UB / dC | Unbalanced load | Off-balance basket, suspension wear | Redistribute load, inspect suspension rods/shocks |
| dE / dC / dL | Door lock error | Bad latch or door not closing | Inspect latch/strike, test lock assembly |
| SUD / 5UD / Sd | Excess suds | Too much or wrong detergent | Run rinse/spin, use HE detergent only |
| HE / HC | Heater error | Heating element or thermistor fault | Check heater and thermistor resistance |
| 3E | Motor error | Hall sensor, stator, inverter issue | Test hall sensor and motor windings |
| 8E / 8C | MEMS/vibration sensor error | Sensor or wiring failure | Check sensor harness and board |
| 9C1 / 9C2 / UC | Voltage error | Bad outlet, extension cord, unstable power | Use dedicated outlet, verify voltage |
| bE / bE2 | Stuck button | Failed keypad or UI board | Test buttons, replace UI if needed |
| tE | Thermistor error | Temp sensor open/shorted | Test thermistor resistance |
| PF | Power failure | Interruption during cycle | Restart cycle and verify power source |

Samsung notes that many washer codes can be cleared by unplugging the unit for several minutes before restarting.

Samsung Dryer Error Codes

| Code | Meaning | Most Common Causes | Recommended Fix |
| --- | --- | --- | --- |
| HE / HC | Overheating | Restricted vent, heater issue | Clean venting, test heater and thermistor |
| tE / tO | Thermistor error | Faulty temp sensor | Test and replace thermistor |
| dE | Door error | Door switch or latch issue | Inspect switch and latch |
| 3E | Motor error | Drive motor or inverter fault | Check motor winding resistance |
| 9C1 / 9C2 | Voltage problem | Improper supply voltage | Verify outlet and breaker |
| FE | Frequency error | Incorrect power frequency | Check power supply |
| AC / AE | Communication error | Main board/UI board communication fault | Inspect harness and boards |
| Et | EEPROM/control issue | Corrupted control memory | Power reset or replace PCB |
| CL9 | Vent blockage warning | Poor airflow | Clean vent system thoroughly |

Vent restriction is one of the biggest causes of Samsung dryer failures and HE/HC overheating codes.

Samsung Refrigerator Error Codes

| Code | Meaning | Most Common Causes | Recommended Fix |
| --- | --- | --- | --- |
| 22E / 22C | Fridge fan error | Ice buildup, failed evaporator fan | Defrost unit, test fan motor |
| 33E | Ice pipe heater error | Heater open | Check heater continuity |
| 40E / 40C | Ice maker fan error | Frozen fan or failed motor | Defrost and test fan |
| 41E | Ice maker communication error | PCB or ice maker fault | Inspect harness and control |
| 84C | Compressor lock | Compressor startup issue | Check inverter board and compressor |
| 86E | Inverter compressor fault | Inverter PCB issue | Test inverter output |
| PC ER | Door communication error | Loose connector at top hinge | Reseat door harness |
| OF OF / O FF | Cooling off mode | Demo mode enabled | Disable showroom/demo mode |
| 5E / SE | Defrost sensor error | Open thermistor | Test sensor resistance |
| RD | Damper error | Stuck air damper | Inspect damper motor |

Samsung Dishwasher Error Codes

| Code | Meaning | Most Common Causes | Recommended Fix |
| --- | --- | --- | --- |
| LC / LE | Leak detected | Water in base tray | Dry base, inspect leak source |
| 4C / 4E | Water supply issue | Low water or closed valve | Check inlet hose and valve |
| 5C / 5E | Drain error | Blocked drain or pump | Clear hose and filter |
| OC / OE | Overflow error | Overfill or float switch issue | Inspect float and inlet valve |
| HE | Heater error | Failed heater or thermistor | Test heating circuit |
| 3E | Wash motor error | Circulation pump failure | Test circulation motor |
| 9E | Water level issue | Pressure sensor fault | Inspect sensor and hose |
| bE | Button stuck | UI panel issue | Check keypad |
| PE | Half-load diverter error | Diverter motor issue | Inspect diverter assembly |

Samsung specifically notes LC/LE codes are commonly caused by moisture in the bottom tray, even from minor leaks or oversudsing.

Samsung Oven / Range Error Codes

| Code | Meaning | Most Common Causes | Recommended Fix |
| --- | --- | --- | --- |
| SE | Keypad error | Shorted touchpad | Replace membrane or UI |
| C-21 | Oven overheating | Cooling fan issue | Check cooling fan operation |
| C-d0 | Door lock error | Door switch/latch fault | Test latch assembly |
| E-08 | Oven sensor issue | Faulty temp sensor | Test RTD sensor |
| tE | Temp sensor fault | Open or shorted probe | Replace oven sensor |
| bE | Button stuck | Control panel issue | Test UI panel |
| LE | Communication error | Main PCB communication issue | Inspect harnesses |
| C-F0 | EEPROM/control fault | PCB corruption | Replace control board |

Samsung Microwave Error Codes

| Code | Meaning | Most Common Causes | Recommended Fix |
| --- | --- | --- | --- |
| SE | Keypad short | Failed membrane switch | Replace control panel |
| E-11 | Humidity sensor error | Sensor fault | Replace humidity sensor |
| E-24 | Temperature sensor issue | Open/shorted thermistor | Test sensor |
| C-d0 | Door sensing issue | Door switch fault | Test interlock switches |
| C-10 | Communication error | PCB communication failure | Inspect harnesses |

General Samsung Diagnostic Tips

- Unplug appliance for 2–5 minutes first.

- Check wiring harnesses before replacing boards.

- Samsung frequently uses:

  - Hall sensors on washers

  - Thermistors on dryers/refrigerators

  - Leak sensors in dishwashers

- Many Samsung appliances store hidden diagnostic history accessible through service mode.

- Oversudsing causes a surprising number of false washer and dishwasher codes.