MAKE SURE TO ASSOCIATE ALL WHIRLPOOL FAMILY BRANDS WITH WHIRLPOOL

Here’s a practical technician-style reference for common Whirlpool appliance error codes, what they mean, and the most common fixes. Whirlpool also owns/uses similar platforms in Maytag, KitchenAid, Amana, and some JennAir products, so you’ll often see similar fault structures across brands.

Whirlpool Washer Error Codes

| Code | Meaning | Most Common Causes | Recommended Fix |
| --- | --- | --- | --- |
| F0 E1 / rL | Load detected during clean cycle | Clothes left in washer | Empty washer and rerun cycle |
| F0 E2 | Oversuds condition | Too much detergent | Run rinse/spin, use HE detergent |
| F1 E1 | Main control failure | Corrupted CCU/main board | Power reset, replace control if persistent |
| F1 E2 | Motor control fault | MCU or wiring issue | Inspect harnesses and motor control |
| F2 E1 | Stuck key | UI/control button stuck | Inspect console and keypad |
| F3 E1 | Pressure system fault | Pressure hose or sensor issue | Check hose for blockage/kinks |
| F5 E2 | Door lock error | Failed latch or strike | Test/replace door lock |
| F5 E3 | Door unlock error | Latch stuck locked | Inspect latch assembly |
| F7 E1 | Motor speed sensing error | Bad actuator, capacitor, or motor | Test motor and shift actuator |
| F8 E1 / LO FL | Long fill/no fill | Low water supply or clogged inlet | Check valves/screens |
| F8 E3 | Overflow condition | Pressure sensor or inlet valve stuck | Unplug washer, inspect valves |
| F9 E1 | Long drain | Clogged drain filter or weak pump | Clean filter and test pump |
| Sud / Sd | Excess suds | Wrong detergent | Flush unit and use HE detergent |
| Int | Cycle interrupted | Lid/door opened or control interruption | Restart cycle |

Whirlpool’s official documentation lists F-series washer faults heavily tied to pressure systems, door locks, and motor sensing.

Whirlpool Top-Load Specific Codes

| Code | Meaning | Common Causes | Recommended Fix |
| --- | --- | --- | --- |
| OL | Overload | Basket overloaded | Reduce load |
| UL | Unbalanced load | Suspension wear or off-balance load | Redistribute load, inspect suspension |
| LF | Long fill | Low incoming water | Check supply pressure |
| LD | Long drain | Drain restriction | Inspect hose and pump |
| HC | Hot/cold reversed | Hoses reversed | Correct hose connections |
| dU | Lid unlock issue | Lid latch problem | Inspect lid lock |
| oL | Overloaded basket drag | Gearcase or clutch drag | Check basket movement |

Whirlpool Dryer Error Codes

| Code | Meaning | Most Common Causes | Recommended Fix |
| --- | --- | --- | --- |
| AF / Check Vent | Restricted airflow | Clogged vent | Clean venting completely |
| PF | Power failure | Voltage interruption | Restart cycle |
| F01 | Main control failure | Failed control board | Replace control |
| F02 | Keypad/UI issue | Stuck button | Inspect console |
| F20 | Heater relay problem | Heater relay/control fault | Check board and heater circuit |
| F22 | Exhaust thermistor open | Thermistor failure | Test thermistor |
| F23 | Exhaust thermistor shorted | Sensor short | Replace thermistor |
| F25 | Inlet thermistor issue | Temp sensor fault | Test sensor |
| F28 / F29 | Moisture sensor issue | Sensor harness/open circuit | Clean and test moisture strips |
| F3 E1 | Thermistor/open fault | Temp sensing issue | Replace thermistor |
| F4 E3 | Restricted airflow overheating | Blocked vent | Clean ducting |
| L2 | Low/no line voltage | Supply voltage issue | Verify 240V supply |
| FC E1 | Wrong power source | Miswired outlet | Correct wiring |

Whirlpool dryers commonly throw AF/F4E3 airflow-related faults from partially restricted vents rather than failed parts.

Whirlpool Refrigerator Error Codes

| Code | Meaning | Most Common Causes | Recommended Fix |
| --- | --- | --- | --- |
| E0 / F0 | Power loss/open door | Outage or prolonged open door | Reset and monitor |
| E1 / F1 | Refrigerator thermistor fault | Bad temp sensor | Test sensor resistance |
| E2 / F2 | Freezer sensor/fan issue | Evap fan or thermistor fault | Check evaporator fan |
| E3 / F3 | Defrost sensor issue | Failed defrost thermistor | Test thermistor |
| E5 / F5 | Ice maker fault | Ice maker heater or sensor issue | Inspect ice maker assembly |
| E6 / F6 | Communication error | UI/main board communication | Check harness/connectors |
| E9 / F9 | High freezer temp | Cooling issue | Check sealed system and airflow |
| PO | Power outage | Recent power interruption | Acknowledge/reset |
| SY / CE | Communication failure | UI or main PCB issue | Inspect harnesses |
| rd | Defrost heater issue | Heater open circuit | Test defrost heater |
| DF | Defrost failure | Heater, board, or thermistor | Check defrost circuit |

Whirlpool refrigerator faults often center around thermistors, evaporator fans, and communication issues between the UI and main control.

Whirlpool Dishwasher Error Codes

| Code | Meaning | Most Common Causes | Recommended Fix |
| --- | --- | --- | --- |
| F2 E2 | UI disconnected | Loose ribbon cable | Reseat UI connections |
| F4 E3 | Wash motor not running | Failed circulation pump | Test wash motor |
| F5 E1 | Door stuck open | Door latch issue | Inspect latch |
| F6 E4 | Float switch open | Float stuck or leak | Inspect float tray |
| F7 E1 | Heating issue | Heater not operating | Check heater circuit |
| F8 E2 | Drain motor electrical issue | Drain pump fault | Test drain pump |
| F8 E4 | Overflow/float activated | Water in base pan | Dry base and locate leak |
| F9 E1 | Long drain | Blocked drain | Clean hose/filter |

A lot of Whirlpool dishwashers enter continuous drain mode when the float switch detects water in the base tray.

Whirlpool Oven / Range Error Codes

| Code | Meaning | Most Common Causes | Recommended Fix |
| --- | --- | --- | --- |
| F1 E0 | EEPROM/control error | Corrupted control board | Replace control |
| F2 E0 | Shorted keypad | Failed touchpad | Replace UI |
| F3 E0 | Oven sensor open | Open RTD sensor | Test sensor resistance |
| F3 E1 | Sensor shorted | Shorted RTD | Replace sensor |
| F5 E1 | Door latch failure | Latch motor or switch issue | Inspect latch assembly |
| F6 E0 | Lost communication | UI/control mismatch | Check harness and boards |
| F8 E0 | Cooling fan speed issue | Failed cooling fan | Test fan motor |

Whirlpool oven faults are heavily dominated by keypad failures, RTD sensor faults, and door latch problems during self-clean cycles.

Whirlpool Microwave Error Codes

| Code | Meaning | Most Common Causes | Recommended Fix |
| --- | --- | --- | --- |
| F1 | EEPROM/control fault | Failed control board | Replace PCB |
| F2 | Keypad short | Shorted membrane switch | Replace control panel |
| F6 | Communication error | UI/main PCB issue | Inspect harness |
| Door | Door switch fault | Failed interlock switch | Test switches |
| SE | Stuck keypad | Shorted keypad membrane | Replace touchpad |

Whirlpool Tech Notes / Common Patterns

Common Whirlpool Failures

- Washer:

  - Shift actuators

  - Lid locks

  - Drain pumps

  - Suspension rods

- Dryer:

  - Thermal fuses

  - Heating elements

  - Restricted venting

- Refrigerators:

  - Evaporator fan motors

  - Ice maker systems

  - UI/main board communication

- Dishwashers:

  - Sump/wash pumps

  - Float switch leak detection

  - Heating circuits

- Ovens:

  - Touchpads

  - RTD sensors

  - Door latch assemblies

Whirlpool Diagnostic Tips

- Unplug appliance for 5 minutes before deeper diagnosis.

- Whirlpool heavily uses:

  - Thermistors

  - Hall/motor speed sensing

  - Pressure switches

  - UI-to-main-board serial communication

- Many Whirlpool products have hidden diagnostic/service modes.

- F# E# codes usually break down as:

  - F = fault category

  - E = subsystem detail